TO-17 Sorbent Cartridge Selection Guide

This is a guide to common sorbent tubes used to collect air samples for EPA Method TO-17.

	Tube Name
	Compounds
	Packing
	Desorption Temperature

	Tenax TA
	BTEX

Diesel Range Organic
	Tenax TA
	300C

	Carbotrap 300
	General VOC
	Carbopak C

Carbopak B

Carboseive SIII
	325C

	VOC
	General VOC
	Tenax TA

Carboxen 1000

Carboseive SIII
	325C


Passive Samplers

TO-17 tubes can also be sampled passively using special adapters.  The tubes are desorbed and analyzed in the same manner as the normal TO-17, and the TO-17 QC criteria is used.

