TO-3/TO-14 BTEX Oxygenates Target List and MDL

Method TO-3 and TO-14 FID use cryogenic trapping and a gas chromatograph with a flame ionization detector (FID) to measure volatile organic compounds collected in Summa Canisters or Tedlar Bags.  This method is suitable for a wide range of hydrocarbons and other volatile compounds having boiling points in the range of –10 to 200C.  

	
	
	
	

	Analyte
	
	MDL

ppbV
	LOQ

ppbV

	Benzene
	
	1
	1

	Toluene
	
	1
	1

	Ethyl Benzene
	
	1
	1

	m & p Xylene
	
	1
	1

	o -Xylene
	
	1
	1

	Ethanol
	
	0.5
	1.5

	Tert butyl alcohol
	TBA
	0.5
	1.5

	Methyl tert butyl ether
	MTBE
	0.5
	1.5

	Diisopropyl ether
	DIPE
	0.5
	1.5

	Ethyl tert butyl ether
	ETBE
	0.5
	1.5

	Tert amyl methyl ether
	TAME
	0.5
	1.5

	TPH (gasoline)
	
	10
	10

	TPH (Diesel)
	
	10
	10


